|  | # Risk Assessment |
| --- | --- |
| Course/Road(s) Assessed: NC14 ‘Valley Course’ | Course: Local to Rockingham Forest Wheelers |
| ## Date of Assessment/Review: 29-04-2025 | Name of Assessor: Phil Wilkinson |

Course Description: A 9.6 mile, anti-clockwise circuit on local country roads, used regularly for club events. START at the National Speed Limit sign exiting Drayton village proceed to Medbourne to join the B664 (Take care). Keep BEARING LEFT on the B664 ignoring all junctions from the right, before BEARING LEFT (leaving the B664) to Ashley (Taking extreme care re Parked Cars and Bends etc.). Continue through Ashley village and continue to Middleton where TURN LEFT at the T-junction to join the B670. Proceed through Middleton (Taking care re Parked Cars etc.) and at the Y-junction in Cottingham leave the B670 and BEAR LEFT towards Bringhurst to the T junction just outside the village (Extreme Care) where TURN LEFT and proceed towards Drayton to finish at the 30mph sign before the village. (approx.9.6 miles)

Traffic Flows: Primarily local traffic, which may include slow-moving farm vehicles. There may be ridden horses on the course,

Course/Event History: The course has been used for many years, with no recent incidents, in 2024 Drayton introduced a 20mph limit though the village and the only alternative to continue using this course was to start and finish either side of the village.

Key Identified Risks

| Distance | ### Location | Identified Significant Risk/Hazards | Level of Risk Low/Med/High | #### Measures to reduce Risk (if applicable) |
| --- | --- | --- | --- | --- |
| 0 miles | Drayton Start / Finish area | Infrequent traffic approaching start / finish area in both directions | Low | Start / Finish area located on straight road with good sighting. Adjacent to National speed limit signs. Cycle Event and Start signs. |
| 2 miles | Medbourne Junction | Local traffic from B664 Uppingham Road joins course from Right at grass triangle Course now runs on B664 | Low | Rider vigilance and discretion |
| 2.1 miles | Neville Arms, Medbourne | Infrequent vehicles from Neville Arms car park on Right | Low | Rider vigilance and discretion |
| 2.3 miles | Paynes Lane, Medbourne | Infrequent traffic joining course from Paynes Lane on Right | Low | Rider vigilance and discretion |
| 3.5 miles | Ashley Junction | Course leaves B664 at Left turn to Ashley. Traffic from Weston by Welland may turn across course. | Low | Rider vigilance and discretion |
| 3.5 – 4 miles | Through Ashley village | Parked vehicles causing impediment to traffic flow in both directions. Traffic emerging from residential roads and drives | Low | Rider vigilance and discretion |
| 5 miles | Wire Lane | Minor lane joins course from Right | Low | Cycle Event sign facing traffic emerging from lane. |
| 7 miles | Middleton Junction At bottom of Middleton Hill | Course joins local road (left-hand turn). Parked vehicles may obscure sighting | Medium | Cycle Event signs on local road in both directions. Marshal at junction. CTT direction arrow for riders. |
| 7-7.5 miles | Through Middleton village | Parked vehicles causing impediment to traffic flow in both directions. Traffic emerging from residential roads and drives | Low | Rider vigilance and discretion |
| 7.5 miles | Cottingham Junction | Course leaves local road at triangular junction at Mill Lane (left –hand turn) . Traffic may merge from the Right | Low | Rider vigilance and discretion |
| 9 miles | Bringhurst Junction | Course joins local road at grass triangle (left-hand turn). | Medium | Cycle Event signs in both directions on local road. CTT direction arrow for riders. Marshal at junction. |
| 9.6miles | Drayton Finish at point adjacent to path end before 30mph sign giving riders 20 meters to slow before new speed limit area. | Infrequent traffic approaching start / finish area in both directions | Low | Start / Finish area located on straight road with good sighting. Cycle Event and Start signs |

NOTES: 1. The use of a vehicle for the timekeeper(s) at start and finish is where appropriate and is only identified as an example for the action to be taken 2. The small junctions or entrances to farms/ facilities (garage,.eatery, etc) that are not identified in this risk assessment have been considered, however are not considered significant to pose a risk and therefore have not been noted.

Date of original Assessment:   not known